VistaMax network protocols and requirements

For

Ethereal plug-in dissector.

Copyright Harris Corp. 2005

The VistaMax system uses TCP/IP to carry commands and status for various parts of the system. This document describes the protocols and specifies an Ethereal plug-in dissector for test and debug of the systems.

There are two custom protocols layered on top of TCP/IP. First, the VistaMax system devices (consoles and routing racks) use the VistaMax Device Control Protocol (VDCP) to communicate with each other for the purposes of routing audio from a source to a destination. Second, Edge Devices communicate with the hosting VistaMax server using the VistaMax Routing Controller Protocol (VRCP).

VistaMax Device Control Protocol (VDCP)

VDCP has been updated during the development of the system and there exist two basic message types.

Version 1 messages have the following format:

| Name | Size (bytes) | Description |
| --- | --- | --- |
| wMessageLength | 2 | The number of bytes following this bytecount word |
| bMessageID | 1 | Message type identifier (binary) |
| dwParam1 | 4 | First message parameter (binary) |
| dwParam2 | 4 | Second message parameter (binary) |

The message parameters are described for each of the given message types.

| CNP_SRC_ATTACH | 0 (0x00) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_SRC_DETACH | 1 (0x01) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_SRC_LOST | 2 (0x02) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_SRC_BY_PROXY | 3 (0x03) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_TELL_ME_CHANGE | 6 (0x06) |
| --- | --- |
| dwParam1 | Source Signal ID (note 1) |
| dwParam2 | Destination Signal ID |

| CNP_SRC_ATTACH_ACK | 100 (0x64) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_SRC_DETACH_ACK | 101 (0x65) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_SRC_BY_PROXY_ACK | 102 (0x66) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_ROUTE_CHANGED | 106 (0x6A) |
| --- | --- |
| dwParam1 | Source Signal ID |
| dwParam2 | Destination Signal ID |

| CNP_WHO_ARE_YOU | 202 (0xCA) |
| --- | --- |
| dwParam1 | Dummy value (note 2) |
| dwParam2 | Dummy value (note 2) |

| CNP_I_AM | 203 (0xCB) |
| --- | --- |
| dwParam1 | Hub Number |
| dwParam2 | Facet Number |

| CNP_WHO_AM_I | 204 (0xCC) |
| --- | --- |
| dwParam1 | Dummy value (note 2) |
| dwParam2 | Dummy value (note 2) |

| CNP_YOU_ARE | 205 (0xCD) |
| --- | --- |
| dwParam1 | Hub Number |
| dwParam2 | Facet Number |

| CNP_COMMAND_CONNECT | 207 (0xCF) |
| --- | --- |
| dwParam1 | Dummy value (note 2) |
| dwParam2 | Dummy value (note 2) |

The following command is the ONLY variable length command in VDCP version 1. There is a single parameter only and that is the filename of the session file to load.

| CNP_LOAD_SESSION | 206 (0xCE) |
| --- | --- |
| Param1 | Filename to load |

Notes:

- To register a destination the dwSource parameter is set to any value other then silence (0xffffffff). To unregister a destination the dwSource parameter is set to silence (0xffffffff).

- Commands that have “Dummy Value” for a parameter ignore the contents of the parameter. The value should be zero but is not guaranteed to be so.

Version 2 messages have the following basic format, a header followed by 1 or more parameter blocks.

| Name | Size (bytes) | Description |
| --- | --- | --- |
| wMessageLength | 2 | The number of bytes following this bytecount word |
| bMessageID | 1 | Message type identifier |
| dwParam.1.1 | 4 | Parameter Block 1, parameter 1 |
| dwParam.1.2 | 4 | Parameter Block 1, parameter 2 |

.

.

.

| dwParam.n.1 | 4 | Parameter Block n, parameter 1 |
| --- | --- | --- |
| dwParam.n.2 | 4 | Parameter Block n, parameter 2 |

| CNP_SRC_ATTACH_LINKED | 5 (0x05) |
| --- | --- |
| Param.1.1 | Source Signal ID |
| Param.1.2 | Destination Signal ID |

.

.

.

| Param.n.1 | Source Signal ID |
| --- | --- |
| Param.n.2 | Destination Signal ID |

| CNP_SRC_DETACH_LINKED | 7 (0x07) |
| --- | --- |
| Param.1.1 | Source Signal ID |
| Param.1.2 | Destination Signal ID |

.

.

.

| Param.n.1 | Source Signal ID |
| --- | --- |
| Paramn.n.2 | Destination Signal ID |

| CNP_SRC_ATTACH_LINKED_ACK | 105 (0x69) |
| --- | --- |
| Param.1.1 | Source Signal ID |
| Param.1.2 | Destination Signal ID |

.

.

.

| Param.n.1 | Source Signal ID |
| --- | --- |
| Param.n.2 | Destination Signal ID |

| CNP_SRC_DETACH_LINKED_ACK | 107 (0x6B) |
| --- | --- |
| Param.1.1 | Source Signal ID |
| Param.1.2 | Destination Signal ID |

.

.

.

| Param.n.1 | Source Signal ID |
| --- | --- |
| Param.n.2 | Destination Signal ID |

VistaMax Routing Controller Protocol (VRCP)

VRCP messages all have a header with the following format:

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Message Length | 2 | The number of bytes following this bytecount word (binary) |
| Request Type | 2 | Type of Request (ASCII) |
| MAC Address | 6 | MAC address of the controller (binary) |
| Unit | 2 | Zero-based logical unit number (binary) |
| Status Code | 2 | Zero-based logical unit number (binary) |
| Message ID | 4 | Unique message ID (binary) |

The following status values may appear in the request message header:

| Value | Description |
| --- | --- |
| 0 | Status is good, no error |
| 1 | Route in progress |
| 2 | Resource Busy |
| 3 | No license available |
| 4 | Out of resources |

Request Types

Each of the following request types has the above header as the first element of the request. The “Request Type” field for each as a two character ASCII type field. Each response message is prefixed by the full request message including the header. Note that the “Message Length” field of the prefixed request must be updated to include the size of the response block.

GetState (GS)

Request

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Header | 18 | Header with “GS” Request type field |
| Destination Name | 10 | Name of Destination (ASCII) |
| Destination Signal ID | 4 | Destination Signal Token (binary) |
| Source Names | 1 | “T” == source, “F” == destination |

Response

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Request Message | 33 | GetState request block |
| Destination Name | 10 | Name of Destination (ASCII) |
| Destination Signal ID | 4 | Destination Signal Token (binary) |
| Source Name | 10 | Name of Source (ASCII) |
| Source Signal ID | 4 | Source Signal Token (binary) |
| Button 1 Name | 10 | Signal Name for Button 1 |
| Button 1 Signal ID | 4 | Signal Token for Button 1 |
| Button 2 Name | 10 | Signal Name for Button 2 |
| Button 2 Signal ID | 4 | Signal Token for Button 2 |
| Button 3 Name | 10 | Signal Name for Button 3 |
| Button 3 Signal ID | 4 | Signal Token for Button 3 |
| Button 4 Name | 10 | Signal Name for Button 4 |
| Button 4 Signal ID | 4 | Signal Token for Button 4 |
| Button 5 Name | 10 | Signal Name for Button 5 |
| Button 5 Signal ID | 4 | Signal Token for Button 5 |
| Button 6 Name | 10 | Signal Name for Button 6 |
| Button 6 Signal ID | 4 | Signal Token for Button 6 |

GetDestList (DL)

Request

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Header | 18 | Header with “DL” Request type field |
| Destination Name | 10 | Name of Destination (ASCII) |
| Destination Signal ID | 4 | Destination Signal Token (binary) |
| Start Offset | 4 | Offset - Anchor to start (+/- binary)) |
| End Offset | 4 | Offset - Anchor to end (+/- binary)) |
| Include All | 1 | “T” or “F” (ASCII) |

Response

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Request Message | 41 | GetDestList request block |
| Destination Name | 10 | Signal Name for Destination (ASCII) |
| Destination Signal ID | 4 | Signal Token for Dest 1 (binary) |

.

.

.

| Destination Name | 10 | Signal Name for Destination (ASCII) |
| --- | --- | --- |
| Destination Signal ID | 4 | Signal Token for Destination (binary) |

GetSourceList (SL)

Request

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Header | 18 | Header with “SL” Request type field |
| Destination Name | 10 | Name of Destination (ASCII) |
| Destination Signal ID | 4 | Destination Signal Token (binary) |
| Start Offset | 4 | Offset - Anchor to start (+/- binary)) |
| End Offset | 4 | Offset - Anchor to end (+/- binary)) |
| Include All | 1 | “T” or “F” (ASCII) |

Response

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Request Message | 41 | GetSourceList request block |
| Source Name | 10 | Signal Name for Source (ASCII) |
| Source Signal ID | 4 | Signal Token for Source (binary) |

.

.

.

| Source Name | 10 | Signal Name for Source (ASCII) |
| --- | --- | --- |
| Source Signal ID | 4 | Signal Token for Source (binary) |

KeepAlive (KA)

Request

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Header | 18 | Header with “KA” Request type field |

Response

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Request Message | 18 | Keep Alive request block |

SetRoute (SR)

Request

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Header | 18 | Header with “SR” Request type field |
| Destination Name | 10 | Name of Destination (ASCII) |
| Destination Signal ID | 4 | Destination Signal Token (binary) |
| Source Name | 10 | Name of Source (ASCII) |
| Destination Signal ID | 4 | Source Signal Token (binary) |

Response

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Request Message | 46 | SetSource request block |
| Source Name | 10 | Signal Name for Source (ASCII) |
| Source Signal ID | 4 | Signal Token for Source (binary) |

SetState (SS)

Request

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Header | 18 | Header with “SS” Request type field |
| State Bits | 4 | List of State Bits (binary) |
| State Bit Mask | 4 | List of bits having meaning here (binary) |

Response

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Request Message | 26 | SetState request block |
| State Bits | 4 | List of State Bits (binary) |

The bit assignments for the SetState request (and Alert) are:

| Bit | Description |
| --- | --- |
| 0 | Privacy state (1 == enabled) |

Alert Messages

The following messages may be sent as alerts to the controller. Alerts have no response messages. Each alert has a header block.

Alert Header

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Alert Type | 2 | Type of Alert (ASCII) |
| MAC Address | 6 | MAC Address of controller (binary) |
| Unit | 4 | Zero-based logical unit number (binary) |

ResetRCED (RR)

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Alert Header | 12 | Alert header with “RR” type field |

ResetSource (RS)

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Alert Header | 12 | Alert header with “RS” type field |
| Destination Name | 10 | Name of Destination (ASCII) |
| Destination Signal ID | 4 | Destination Signal Token (binary) |
| Source Name | 10 | Name of Source (ASCII) |
| Destination Signal ID | 4 | Source Signal Token (binary) |

SetState (SS)

| Name | Size (bytes) | Description |
| --- | --- | --- |
| Alert Header | 12 | Alert header with “SS” type field |
| State Bits | 4 | List of state bits (binary) |
| State Bit mask | 4 | List of bits active here (binary) |

Ethereal Dissector

A plug-in dissector is required for VDCP and VRCP. The plug-in should permit the selection of either protocol for parsing. Having both protocols active at the same time would be desirable but it is not essential.

Protocol Ports

The port that the VDCP server listens on is 6010, note that this is configurable for VistaMax systems; to date it has not been changed. It is desirable that the dissector read the port value from a text file so that should a change be required it may be done without re-compiling the dissector.

The port that the VRCP server listens on is 4001; again this is configurable and the dissector should acquire the value from a text file.

Should either of the above port numbers be missing from the text file, or the text file itself not be present the dissector should use the given default values.

Development Environment

So that the dissector plug-in may be maintained for future protocol additions and changes it is most desirable that it be developed with Microsoft tools. Both Visual Studio .NET 2003 and Visual Studio .NET are available for this maintenance.

Deliverables

Initially:

- Cost estimate for implementation of dissector plug-in

- Time estimate (elapsed) for delivery

- Timeline for development with measurable goals.

Upon project execution:

- Ethereal plug-in dissector as described (Dynamic Link Library for Win32)

- Full annotated source code. (Microsoft VS.Net compatible)

- Brief descriptive document outlining the structure and operation of the code.

Note: All rights to the dissector, including source code, binary code, and documentation become the sole property of Harris Corp.