#### Reference: PacketCable Dynamic Quality of Service Specification, pkt-sp-dqos-I03-020116 (This specification is available at www.packetcable.com)

Gate control messages are carried over COPS interface.

The following table lists the Gate Control Commands supported by PacketCable for Gate control messages.

| Command Type | Command Code | Description |
| --- | --- | --- |
| GATE-ALLOC | 1 |  |
| GATE-ALLOC-ACK | 2 |  |
| GATE-ALLOC-ERR | 3 |  |
| GATE-SET | 4 |  |
| GATE-SET-ACK | 5 |  |
| GATE-SET-ERR | 6 |  |
| GATE-INFO | 7 |  |
| GATE-INFO-ACK | 8 |  |
| GATE-INFO-ERR | 9 |  |
| GATE-DELETE | 10 |  |
| GATE-DELETE-ACK | 11 |  |
| GATE-DELETE-ERR | 12 |  |

The following table lists gate control messages and required attributes:

| Messages | COPS objects of the message |
| --- | --- |
| GATE-ALLOC | -Decision Header -Transaction ID -Subscriber ID -Activity Count |
| GATE-ALLOC-ACK | -ClientSI Header -Transaction ID -Subscriber ID -Gate-ID -Activity Count -Gate Coordination Port |
| GATE-ALLOC-ERR | -Client Header -Transaction ID -Subscriber ID -PacketCable Error |
| GATE-SET | -Decision Header -Transaction ID -Subscriber ID - Gate-ID -Activity Count -Remote Gate Info -Event Generation Info -Media Connection Event Info -Electronic Surveillance Parameters -Session Description Parameters -Gate Spec -[Gate Spec] |
| GATE-SET-ACK | -ClientSI Header -Transaction ID -Subscriber ID -Gate-ID -Activity Count -Gate Coordination Port |
| GATE-SET-ERR | -Client Header -Transaction ID -Subscriber ID -PacketCable Error |
| GATE-INFO | -Decision Header -Transaction ID -Gate ID |
| GATE-INFO-ACK | -Client Header -Transaction ID -Subscriber ID -Gate ID -Remote Gate Info -Event Generation Info -Media Connection Event Info -Gate Spec -[Gate Spec] |
| GATE-INFO-ERR | -ClientSI Header -Transaction ID -Gate ID -PacketCable Error |
| GATE-DELETE | -Decision Header -Transaction ID -Gate ID |
| GATE-DELETE-ACK | -ClientSI Header -Transaction ID -Gate ID |
| GATE-DELETE-ERR | -ClientSI Header -Transaction ID -Gate ID -Packetcable error |